HAS-MERA-260922

MERA v1.0 Hardware Architecture Specification

Multi-channel Deterministic RF Event Recorder IP
Revision 1.0 — 2026-09-22

# 0. Document Control

| 항목 | 값 |
| --- | --- |
| Document ID | HAS-MERA-260922 |
| IP Name | MERA — Multi-channel Deterministic RF Event Recorder |
| Version | v1.0 |
| Status | Architecture Baseline / RTL Entry Candidate |
| Target | AMD Versal RF class / FPGA prototype |
| Implementation | HLS C++ + RTL wrapper/critical control as required |
| Primary interfaces | AXI4-Stream input, AXI4-Lite control, AXI4-MM output |
| Primary objective | deterministic multi-channel event capture and record formation |
| Sign-off condition | all MUST requirements in Section 4 pass |

# 1. Scope

MERA는 이미 존재하는 RF ADC/DSP/AIE primitive를 대체하지 않는다. 입력 stream을 이벤트 중심의 multi-channel record로 변환하고, pre-trigger/post-trigger 데이터를 보존하며, timestamp와 상태를 부가하고, DDR로 지속 저장할 수 있는 capture/control IP를 정의한다.

# 2. Non-Goals

RF ADC analog control/Calibration 구현

FFT/channelizer/resampler 구현

AI Engine kernel 구현

generic AXI DataMover 재구현

DDR controller 구현

특정 radar/EMSO detection algorithm 구현

아날로그 trigger comparator 구현

# 3. External Architecture

권장 top-level:

RF/DSP/AIE stream → AXIS Input Adapter → Channel/Alignment → Circular Capture → Trigger/FSM → Record Formatter → Event Queue → AXI-MM Writer → DDR

Control path: AXI4-Lite → CSR block → Trigger/FSM/Writer/Status

# 4. Normative Requirements

| ID | Priority | Requirement |
| --- | --- | --- |
| MERA-REQ-001 | MUST | AXI4-Stream compliant input handshaking |
| MERA-REQ-002 | MUST | Armed 상태에서 정의된 정상 입력률을 buffer가 흡수해야 한다. |
| MERA-REQ-003 | MUST | Trigger timestamp를 64-bit로 latch해야 한다. |
| MERA-REQ-004 | MUST | Pre-trigger와 post-trigger sample 수를 독립적으로 설정할 수 있어야 한다. |
| MERA-REQ-005 | MUST | 활성 채널의 sample index alignment 결과를 record에 기록해야 한다. |
| MERA-REQ-006 | MUST | 완료된 record는 versioned header를 가져야 한다. |
| MERA-REQ-007 | MUST | DDR write failure/overflow를 sticky status로 표시해야 한다. |
| MERA-REQ-008 | MUST | Event ID가 동일 event를 유일하게 식별해야 한다. |
| MERA-REQ-009 | MUST | AXI4-Lite CSR로 ARM/DISARM/TRIGGER/FLUSH가 가능해야 한다. |
| MERA-REQ-010 | MUST | Configuration error를 capture 시작 전에 감지해야 한다. |
| MERA-REQ-011 | SHOULD | 4/8/16 channel parameterization |
| MERA-REQ-012 | SHOULD | 128/256/512-bit AXIS packing 옵션 |
| MERA-REQ-013 | SHOULD | External trigger와 software trigger 동시 지원 |
| MERA-REQ-014 | SHOULD | threshold/event input 지원 |
| MERA-REQ-015 | SHOULD | record-level CRC32 |
| MERA-REQ-016 | MAY | dual-buffer DMA scheduling |

# 5. Parameter Table

| Parameter | Default | Range/Constraint |
| --- | --- | --- |
| N_CH | 8 | 4/8/16 |
| IQ_BITS | 16 | 8~24 |
| AXIS_W | 128 | 32/64/128/256/512 |
| TS_W | 64 | fixed |
| PRE_DEPTH | 65536 | power-of-two |
| POST_DEPTH | 262144 | power-of-two |
| EVENT_Q_DEPTH | 64 | 16~256 |
| TRIG_SOURCES | 4 | >=2 |
| HEADER_BYTES | 64 | fixed v1.0 |
| ADDR_W | 64 | 32/64 |
| MAX_BURST_BEATS | 64 | 2/4/8/16/32/64/128/256; backend dependent |

# 6. Interface Specification

## 6.1 AXI4-Stream Input

| Signal | Dir | Width | Requirement |
| --- | --- | --- | --- |
| ACLK | in | 1 | input clock |
| ARESETN | in | 1 | active-low reset |
| TDATA | in | AXIS_W | sample payload |
| TVALID | in | 1 | source valid |
| TREADY | out | 1 | sink ready |
| TLAST | in | 1 | optional frame boundary |
| TKEEP | in | AXIS_W/8 | byte validity |
| TUSER | in | >=1 | optional channel/event metadata |

v1.0은 TDATA packing convention을 명시적으로 고정한다: [channel][I][Q] 또는 interleaved channel-major. 실제 RTL sign-off 전에 하나를 선택하고 golden model과 일치시킨다.

## 6.2 AXI4-Lite Control

32-bit register data, 32-bit aligned addressing을 기본으로 한다.

## 6.3 AXI4-MM Output

| 항목 | v1.0 |
| --- | --- |
| Direction | M_AXI_S2MM |
| Address | 64-bit |
| Data width | 128-bit baseline |
| Burst | up to 64 beats baseline; backend allows 2~256 |
| Outstanding | parameterized |
| Alignment | record base aligned to AXI width |
| Write policy | record contiguous, header first |

# 7. Internal Block Specifications

## 7.1 AXIS Input Adapter

AXIS handshake를 검증하고 sample beat를 내부 canonical format으로 변환한다. TKEEP/TLAST illegal pattern은 ERROR로 기록한다.

## 7.2 Channel Demux

TUSER 또는 static packing rule에 따라 채널별 sample index를 생성한다.

## 7.3 Sample Counter

capture clock 기준 free-running 64-bit sample counter. Trigger acceptance cycle의 counter 값을 timestamp로 latch한다.

## 7.4 Circular Buffer

각 채널의 최근 PRE_DEPTH samples를 유지한다. write pointer는 modulo power-of-two wrap을 사용한다.

## 7.5 Trigger Engine

EXT, SW, threshold/event input을 OR/priority policy로 조합한다. 이미 POST_CAPTURE 중인 경우 새로운 trigger 처리 정책은 DROP/QUEUE로 설정 가능해야 한다.

## 7.6 Alignment Engine

채널별 offset register와 common sample counter를 이용해 common event index를 생성한다. 허용 범위를 벗어나면 ALIGN_ERROR를 설정한다.

## 7.7 Capture FSM

RESET→IDLE→ARMED→TRIGGERED→POST_CAPTURE→COMMIT→DMA_WAIT→DONE. Error는 별도 sticky state.

## 7.8 Record Formatter

64-byte header와 payload를 결합한다. header version을 통해 향후 포맷 확장을 허용한다.

## 7.9 Event Queue

capture completion descriptor를 FIFO에 넣어 DDR writer와 capture path를 분리한다.

## 7.10 AXI-MM Writer

descriptor를 읽어 header/payload를 burst write한다. DDR contention이 있어도 capture path가 즉시 정지하지 않도록 queueing한다.

## 7.11 Status/Telemetry

overflow, drop, alignment, invalid config, DMA error, event count를 sticky/live 형태로 제공한다.

# 8. Capture State Machine

| State | Output/Action | Transition |
| --- | --- | --- |
| RESET | clear state/pointers/status | ARESETN=1 |
| IDLE | no capture | ARM=1→ARMED |
| ARMED | circular buffer active | trigger→TRIGGERED |
| TRIGGERED | latch timestamp/event id; freeze logical pre-window | next→POST_CAPTURE |
| POST_CAPTURE | collect POST_LEN | count==POST_LEN→COMMIT |
| COMMIT | write descriptor/header metadata | queue available→DMA_WAIT |
| DMA_WAIT | DDR write | complete→DONE; error→ERROR |
| DONE | IRQ/status update | REARM→ARMED / else IDLE |
| ERROR | sticky error | SW CLEAR→IDLE |

# 9. Timing and Determinism

Trigger acceptance latency와 event timestamp의 정의를 분리한다. timestamp는 trigger가 acceptance된 clock cycle의 sample counter 값으로 정의한다. 따라서 trigger source 자체의 asynchronous propagation delay는 외부 시스템 측정 항목이며, IP 내부의 acceptance-to-latch latency는 cycle-count로 고정한다.

| Metric | Definition | Target |
| --- | --- | --- |
| T_accept | trigger valid → trigger accepted | 1~N cycles, source별 고정 |
| T_latch | accepted → timestamp latch | 1 cycle |
| T_post | trigger → post capture complete | POST_LEN samples + fixed control latency |
| T_commit | capture complete → descriptor enqueue | bounded constant |
| T_dma | descriptor → DDR completion | memory/interconnect dependent |
| Alignment error | max ¦CH_i index - common index¦ | 0 sample after configured compensation |

# 10. Buffer Sizing Equations

복소 I/Q interleaved sample의 channel당 byte/sample은 2×IQ_BITS/8. 총 pre-buffer 용량:

B_pre = N_CH × PRE_DEPTH × (2×IQ_BITS/8) bytes

post-buffer 용량:

B_post = N_CH × POST_DEPTH × (2×IQ_BITS/8) bytes

예: N_CH=8, IQ_BITS=16, PRE=65536이면 B_pre = 8×65536×4 = 2,097,152 bytes = 2 MiB.

POST=262144이면 B_post = 8×262144×4 = 8,388,608 bytes = 8 MiB.

# 11. Record Memory Layout

| Field | Size | Encoding |
| --- | --- | --- |
| MAGIC | 4B | ASCII MERA |
| VERSION | 2B | 0x0100 |
| HEADER_LEN | 2B | 64 |
| EVENT_ID | 4B | unsigned |
| CHANNEL_MASK | 4B | bit per channel |
| TIMESTAMP | 8B | unsigned sample time |
| PRE_SAMPLES | 4B | unsigned |
| POST_SAMPLES | 4B | unsigned |
| SAMPLE_WIDTH | 4B | bits |
| SAMPLE_FORMAT | 4B | enum |
| ALIGN_STATUS | 4B | bitfield |
| STATUS | 4B | bitfield |
| PAYLOAD_BYTES | 8B | unsigned |
| RESERVED | 8B | zero |
| PAYLOAD | variable | channel-major or agreed packing |

# 12. CSR Register Specification

| Addr | Name | Access | Bit/Field | Reset | Description |
| --- | --- | --- | --- | --- | --- |
| 0x000 | CONTROL | RW | 0 ENABLE | 0 | enable block |
| 0x000 | CONTROL | RW | 1 ARM | 0 | arm capture |
| 0x000 | CONTROL | RW | 2 SW_TRIGGER | 0 | 1-cycle pulse |
| 0x000 | CONTROL | RW | 3 FLUSH | 0 | clear buffers/queue |
| 0x004 | STATUS | RO | 0 ARMED | 0 | armed |
| 0x004 | STATUS | RO | 1 TRIGGERED | 0 | trigger latched |
| 0x004 | STATUS | RO | 2 CAPTURING | 0 | post capture |
| 0x004 | STATUS | RO | 3 DONE | 0 | record done |
| 0x004 | STATUS | RO | 4 OVERFLOW | 0 | sticky |
| 0x004 | STATUS | RO | 5 ALIGN_ERROR | 0 | sticky |
| 0x004 | STATUS | RO | 6 DMA_ERROR | 0 | sticky |
| 0x008 | TRIGGER_SEL | RW | [3:0] | 0 | source select |
| 0x00C | CHANNEL_MASK | RW | N_CH bits | 0 | active channels |
| 0x010 | PRE_LEN | RW | 31:0 | 0 | pre samples |
| 0x014 | POST_LEN | RW | 31:0 | 0 | post samples |
| 0x018 | TS_LO | RO | 31:0 | 0 | timestamp low |
| 0x01C | TS_HI | RO | 31:0 | 0 | timestamp high |
| 0x020 | EVENT_ID | RO | 31:0 | 0 | current event |
| 0x024 | BASE_LO | RW | 31:0 | 0 | DDR base low |
| 0x028 | BASE_HI | RW | 31:0 | 0 | DDR base high |
| 0x02C | EVENT_Q_DEPTH | RW | 15:0 | 64 | queue depth |
| 0x030 | IRQ_EN | RW | 31:0 | 0 | interrupt enables |
| 0x034 | IRQ_STATUS | RW1C | 31:0 | 0 | interrupt status |
| 0x038-0x054 | ALIGN_OFFSET[i] | RW | signed | 0 | per-channel sample offset |
| 0x058 | CAP_COUNT | RO | 31:0 | 0 | completed captures |
| 0x05C | DROP_COUNT | RO | 31:0 | 0 | dropped events |

# 13. Error Model

| Code | Condition | Action |
| --- | --- | --- |
| E001 | invalid PRE/POST configuration | reject ARM |
| E002 | channel mask empty | reject ARM |
| E003 | buffer overflow | sticky + record status |
| E004 | event queue full | DROP_COUNT++ |
| E005 | alignment mismatch | ALIGN_ERROR |
| E006 | AXI write response error | DMA_ERROR |
| E007 | illegal AXIS TKEEP/TLAST | STREAM_ERROR |
| E008 | timestamp discontinuity | TIMESTAMP_ERROR |

# 14. HLS Partitioning Recommendation

| Block | HLS 적합성 | 이유 |
| --- | --- | --- |
| Record Formatter | 높음 | stream transform + fixed header |
| Channel Packing | 높음 | array/stream processing |
| Trigger FSM | 중간 | simple FSM, HLS 가능 |
| Circular Buffer | 중간 | memory banking/partitioning 제어 필요 |
| AXI-MM Writer | 중간 | burst semantics와 outstanding 제어가 중요 |
| CSR | 낮음~중간 | 얇은 RTL wrapper 권장 |
| Assertions/Protocol | RTL | SVA 및 protocol checker가 적합 |

권장 구현은 '100% HLS'가 아니라 HLS datapath + RTL shell이다. 특히 AXI4-Lite CSR, 정확한 AXI protocol boundary, reset/interrupt, SVA는 RTL로 고정하고, record packing/formatting/copy path는 HLS로 구현하면 설계 의도가 명확하다.

# 15. Verification Plan

| Test ID | Scenario | Expected |
| --- | --- | --- |
| V001 | no trigger for 1M samples | no record |
| V002 | SW trigger at sample 1000 | pre/post exact |
| V003 | trigger at circular wrap | correct wrap reconstruction |
| V004 | trigger immediately after ARM | short/zero pre policy exact |
| V005 | backpressure burst | no illegal AXIS behavior |
| V006 | event storm | queue/drop policy exact |
| V007 | 4/8/16 channel | channel map exact |
| V008 | per-channel offset ±1/±4 | alignment correction exact |
| V009 | DDR write error | DMA_ERROR sticky |
| V010 | reset during capture | no ghost record after reset |
| V011 | max PRE/POST | address range safe |
| V012 | random trigger positions | 10^5 randomized events, bit-exact golden comparison |

# 16. Acceptance Criteria

100,000 random trigger simulation에서 golden model과 payload mismatch = 0

AXI4-Stream protocol assertion failure = 0

invalid configuration이 ARM을 차단

4/8/16 channel parameter build 성공

FPGA target에서 지정 clock constraint에 timing closure

pre-trigger wrap boundary에서 sample loss = 0 under specified buffer margin

alignment test에서 compensation 후 0-sample error

DMA error 및 event drop가 software-visible status로 정확히 보고됨

# 17. FPGA Benchmark Matrix

| Case | Channels | AXIS | Clock | PRE | POST | 측정 |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | 4 | 128 | 250MHz | 4K | 16K | LUT/BRAM/Fmax/latency |
| B2 | 8 | 128 | 250MHz | 64K | 256K | LUT/BRAM/Fmax/latency |
| B3 | 16 | 256 | 250MHz | 64K | 256K | LUT/BRAM/Fmax/latency |
| B4 | 8 | 256 | 500MHz | 64K | 256K | sustained BW |
| B5 | 8 | 512 | 250MHz | 64K | 256K | DDR efficiency |
| B6 | 8 | 128 | 250MHz | 256K | 1M | memory scaling |

# 18. Known Limitations / Open Questions

VR1952의 exact RF-ADC→PL physical interface width/clock/buffering 공개 수치는 본 HAS에서 확정하지 않는다. 제품별 confidential documentation으로 sign-off 필요.

AMD 제품 페이지와 DS950 사이의 Channelizer/FFT count discrepancy는 문서 정의/버전 차이로 보이며, RTL 설계 자원 산정은 DS950 기준으로 한다.

MERA와 기능적으로 동일한 AMD customer/early-access IP 존재 여부는 공개 자료로 확정할 수 없다.

DDR sustained bandwidth는 특정 FPGA board와 memory configuration에서 실측해야 한다.

threshold trigger를 MERA 내부에서 계산할지 AIE/PL upstream detector가 제공할지는 제품 profile에 따라 분리할 수 있다.

# 19. Sign-off Checklist

| 항목 | Owner | Status |
| --- | --- | --- |
| Architecture reviewed | Lead | OPEN |
| Interface packing frozen | RTL/HLS | OPEN |
| CSR map frozen | RTL | OPEN |
| Record format frozen | SW/HLS | OPEN |
| Golden model available | Verification | OPEN |
| C-sim passed | Verification | OPEN |
| Co-sim passed | Verification | OPEN |
| FPGA timing passed | Implementation | OPEN |
| DDR throughput measured | System | OPEN |
| Customer-equivalent benchmark completed | Architecture | OPEN |

# 20. References

AMD Versal RF Series Product Page — https://www.amd.com/en/products/adaptive-socs-and-fpgas/versal/rf-series.html

AMD DS950 — Versal Architecture and Product Data Sheet — https://docs.amd.com/api/khub/documents/BY0FMkLH05y85Nwu4GmWsw/content

AMD DS945 — Versal XQ Architecture and Product Data Sheet — https://docs.amd.com/api/khub/documents/wBgkyKdTzQhGgubawOKewA/content

AMD AM009 — AI Engine Architecture Manual: Stream Switch Buffering — https://docs.amd.com/r/en-US/am009-versal-ai-engine/Stream-Switch-Buffering-and-Latency

AMD AM009 — AI Engine to PL Interface — https://docs.amd.com/r/en-US/am009-versal-ai-engine/AI-Engine-to-Programmable-Logic-Interface

AMD AM009 — AXI4-Stream Interconnect — https://docs.amd.com/r/en-US/am009-versal-ai-engine/AXI4-Stream-Interconnect

AMD PG022 — AXI DataMover — https://docs.amd.com/r/en-US/pg022_axi_datamover/Features

AMD/Xilinx RFSoC-MTS GitHub — https://github.com/Xilinx/RFSoC-MTS

AMD/Xilinx RFDC interrupt example — https://github.com/Xilinx/embeddedsw/blob/master/XilinxProcessorIPLib/drivers/rfdc/examples/xrfdc_intr_example.c